Seat No. :  _______________

## AC-118

# April -2018

B.Sc., Sem.-VI

CC-308 : Electronics

Time :  3 Hours]		[Max. Marks : 70

Instructions :	(1)	All the questions carry equal marks.

(2)	Symbols have their own meaning.

1.	(a)	Explain about R-2R type D/A converter. Also draw the circuit of 4 bit D/A converter using input gates, flip-flop, level amplifier & resistive divider network. 	10

OR

Give logic diagram of 3 bit simultaneous type A/D converter & explain it’s working in detail.

(b)	Calculate VA for 5 bit resistive Divider network having input 10110. Here,                0 = 0 V & 1 = 10 V.			4

OR

Calculate the output voltage for binary ladder circuit having digital input 1000. Here, 0 = 0 V & 1 = 10 V.

2.	(a)	Write a program to count 0 to 9 with 2 sec delay between each count. After count 9 it restart to 0 & repeat the sequence continuously. Clock frequency = 0.5 MHz.	10

OR

Write a program to count continuous hexadecimal numbers FFH to OOH in a system of 0.5 s. Use register C to set up 10 ms delay between each count.

(b)	Calculate internal delay and total delay for the following program having 
1 T State = 0.5 s. (any one)		4

(1)		MVI B, 38 H

L-2	MVI C, FF H

L-1	DCR C

JNZ LOOP-1

DCR B

JNZ LOOP-2

(2)		MVI B, 42 H

L-2	MVI C, DD, H

L-1	DCR C

JNZ LOOP-1

DCR B

JNZ LOOP-2

3.	(a)	Write a program to provide the given ON/OFF time to 3 traffic lights (G, Y & R) & two pedestrian signs (walk & Don’t walk).	10

| Lights | Data bits | ON time |
| --- | --- | --- |
| Green | D1 | 20 sec |
| Yellow | D3 | 5 sec |
| Red | D5 | 25 sec |
| Walk | D6 | 20 sec |
| Don’t walk | D7 | 30 sec |

The traffic & pedestrian flow are in same direction and pedestrian should cross the road when green light is on.

OR

Write a 20 ms time delay subroutine using register pair BC. Clear the Z flag without affecting any other flags in the flag register & return to the main program.

(b)	Explain about 8 RST instructions of 8085.	4

OR

Give difference & Similarity between CALL & RET, PUSH & POP.

4.	(a)	Draw the block diagram of 8255A & explain each block in detail. Also explain MODE O as simple input or output.	10

OR

Explain DAC 0808 giving its feature, pin configuration, block diagram & application.

(b)	Explain about control word of IC 8255 A.	4

OR

Write a program to generate square wave.

5.	Answer in short :					14

(1)	________ bit instructions such as DCX & INX do not affect flags.

(2)	Counter & time delay can be designed using ________.

(3)	________ is a group of memory location.

(4)	A large software projected is usually divided into subtasks called ________.

(5)	IC 8255A is a ______ pin DIP.

(6)	What is BSR ?

(7)	What is SAR ?

(8)	What is monotonicity test ?

(9)	Explain about the following instructions :

(i)	CC	(ii)	CZ	(iii)	CP

(iv)	RM	(v)	RP	(vi)	RPE

__________