Seat No. :  _______________

## AA-130

# April-2018

M.Sc. (CA & IT) Integrated, Sem.-VI

Data Communication and Networking

Time :  3 Hours]		[Max. Marks : 100

1.	(A)	Answer the following questions : (any six) 	18

(1)	What is point to point and multipoint connection ?

(2)	What is Dataflow mode ?

(3)	What is Piggybacking ?

(4)	Do Byte stuffing for data “DATA@@ ##COMMUNICATION @@##NETWORKING@#” where@ is flag and # is esc.

(5)	Do Bit stuffing for data “0111 1111 0111 1100 0111 1110” where 0111 1110 is flag.

(6)	Calculate code word for data word where k=3 using even parity bit method.

(7)	205.16.37.36/28   is classless IP address. Find the first address, last address & total number of address.

(B)	Explain Automatic Repeat Request. 	2

2.	(A)	Answer the following questions : (any three) 	18

(1)	Find the CRC for data word 1000 where divisor is 1011.

(2)	Find the 4 bit check sum for data 10,7,6,4,8.

(3)	What is Sliding window protocol ? Explain Selective Repeat sliding window protocol.

(4)	What is Routing ? Explain Link state routing algorithm in detail.

(B)	Explain Bursty Data Rate. 	2

3.	(A)	Answer the following questions : (any three) 	18

(1)	What is Congestion ? Explain close loop in detail.

(2)	Explain Distance vector routing algorithm with example.

(3)	Explain Go Back N sliding window protocol with example.

(4)	Explain Open loop technique for congestion control.

(B)	Explain Negative acknowledgement.	2

4.	(A)	Answer the following questions :	10

(1)	199.10.34.210 is classful IP address. Find the network id and host id.

(2)	Why sender window size is 2n – 1 In Go back N Protocol  ?

(3)	Write difference between connection oriented service verses connection less service.

(4)	Explain Stop & Wait Protocol.

(5)	What is Port address  ? Write different category of Port address.

(B)	Answer the following questions :	10

(1)	What is wavelength and latency ?

(2)	What is bandwidth ?

(3)	There are two antennas; it takes 5 ms to signal to reach from antenna A to antenna B.   Find frequency and wavelength. (c=3 × 108)

(4)	What is virtual circuit identifier ?

(5)	What are categories of coaxial cables and its use ?

5.	(A)	Answer the following question :	18

(1)	What are the transmission media ? Explain about twisted pair cable.

(2)	What is switching network ? Explain packet switching.

(3)	What is multiplexing ? Explain frequency Division multiplexing.

(B)	Explain Time and Frequency Domain. 	2

____________